W1HQ Remote RX System

2/4/2013 – mse

See w1hq-help.dyndns.org for updates.

Requirements

- Remote System

  - BeagleBoard Hardware, power supply, enclosure with audio isolation

  - Ten-Tec Jupiter.  (Other rigs with good CAT control would work)

  - Ubuntu 11.10 (GNU/Linux 3.2.7-x5 armv7l) [and updates]

  - PortAudio from PortAudio.org

  - Speex from Speex.org

  - Rigctld from hamlib.org

  - afxmit.c (custom C source)

  - ddns updater to establish IP address

- Internet Requirements at Remote Site

  - 20-30 kb/s typical in use while running

  - 10/100 Mb/s Ethernet connection via CAT-5

  - Configure Cable/DSL router with two ports passed through firewall

- Local System (@ W1HQ)

  - Windows XP SP3 (“reasonable” CPU, RAM, disk)

  - Soundcard + Speakers and/or phones output

  - afrecv.exe (built with Visual Studio Express 2010 C++ from source)

  - DirectX June 2010 runtime download (only XAudio2 really needed)

  - Visual C++ runtime redistributable download

  - putty and plink for SSH communications (download)

Installation Procedure (Overview)

- Remote Site

  - Provide dry, indoor space for receiver & control system

  - AC power  (possible UPS backup or surge protection)

  - Antenna (broadband HF, non-directional)

  - CAT-5 / CAT-6 connection to router

  - Assign a fixed IP, probably 192.168.1.nnn, for the rig controller (outside DHCP range)

  - Configure local router firewall to pass ports 22 (ssh) and 4532 (rigctld) to the assigned remote rig controller IP

  - Configure rig controller's IP as above, probably using serial port and a laptop.  Verify normal (outgoing) Internet service.

  - Check incoming routing of special ports (as above), using remote rig software at HQ (or elsewhere outside of controller's QTH)

  - Set up & verify DDNS service from controller (via ddclient) to set the proper DNS entry for w1hq.dyndns.org.  Verify operation by rebooting cable modem (which may change IP)

- W1HQ Station Computer

  - Starting with “vanilla” Windows XP SP3 (security patches up to date!)

  - Create “remoterx” directory under C:\Documents and Settings\w1hq

  - Download & Install following software

    - DirectX June, 2010 end-user runtime (49 MB) e.g. :

| Version: | 9.29.1974 | Date published: | 4/18/2011 |
| --- | --- | --- | --- |

    - Visual Studio 2010 C++ redistributable (SP1) (5 MB?)

    - Python 2.7 (latest version, 32 bit ASCII windows) from python.org

Best to add C:\Python27 to system PATH environment variable

    - wxPython 2.8 (latest version for Python 2.7 32 bit)
e.g. wxPython2.8-win32-ansi-py27

    - Putty & Plink (SSH desktop & command-line clients) e.g. beta 0.62

      - Install manually in C:\Program Files\putty

      - Add C:\Program Files\putty to environment PATH variable

      - (optional) create desktop shortcut for putty

  - afrecv.exe (C++ sound receiver, production version – not debug)

    - Place in remoterx\af directory

  - remote.py (Python control panel software)

    - Place files remote.py, Fspin.py, hamlibio.py, smeter.py and Signal_Meter_M0MTJ_1.png in remoterx

    - Create desktop shortcut for remote.py (or remoterx directory)

  - Nircmdc.exe (Windows command utility - http://www.nirsoft.net/utils/nircmd.html)

    - Place in remoterx directory.

File Manifest for Python/C++ local software

- Folder af

  - afrecv (Linux executable, compiled for i386 or amd64)

  - afrecv.exe (Windows, 32 bit)

  - Source files

    - af.h

    - afdump.c

    - afrecv.c

    - afxmit.c

    - gccdump

    - gccrecv

    - gccxmit

    - readme.txt

- FSpin.py

- hamlibio.py

- indicator-grn.png

- Note.txt

- remlab.bat

- remlab.sh

- remote.bat

- remote.py (main program, current version 0.27?)

- remote.sh

- Signal_Meter_M0MTJ_1.png

- smeter.py

- nircmdc.exe

- w1hq_remote.doc

- ??

Problems and areas for future work

- (Possible) occasional hang-up of audio (goes silent). Work-around: try “Stop” then “Start”. Or kill program and re-start.

- (Possible) occasional hang-up of control link (LED stops blinking). Work around: kill program and restart.

- Slow “Start” and “Stop” of audio stream (for T/R switching, e.g.). Not a problem in Linux, but Windows is slow to create subprocesses that are used for these functions. Eventually: set up local TCP/IP channel for Python to control afrecv mute function. Or set up interface with Windows audio control panel for muting.

- Audio compression is not implemented for Windows version. Raw audio rate is about 20 kB/s, which is higher than desired on a remote Internet circuit. Eventually: Implement Opus codec on Linux and WIndows.